ARDUINO PROJECT DOCUMENT

# Smart Crowd Counting and Overcrowding Alert System

A four-sensor entry and exit counter with visual crowd-level status

# Project overview

This project uses an Arduino Uno and four IR sensors to count people entering and leaving a monitored area. Ordered detector and confirmer readings validate each movement within five seconds. A status RGB LED displays the crowd range, while a separate LED signals accepted counts and rejected sequences.

# Objective

To build a simple embedded system that tracks the number of people in an area, indicates the current crowd level, and provides a visible alert when the count reaches 20.

# Components required

| Component | Quantity / specification |
| --- | --- |
| Arduino Uno | 1 |
| IR obstacle sensors | 4 |
| RGB LED | 1, common cathode |
| Red blink/count LED | 1 |
| 220 ohm resistors | 4 |
| Breadboard and jumper wires | As required |

# Pin and connection table

| Signal / component | Arduino connection | Purpose |
| --- | --- | --- |
| E1 | D2 | Entry detector |
| E2 | D3 | Entry confirmer |
| X1 | D4 | Exit detector |
| X2 | D5 | Exit confirmer |
| BLINK_LED | D8 through 220 ohm resistor | Count / rejection indication |
| RED | D9 through 220 ohm resistor | RGB red channel |
| GREEN | D10 through 220 ohm resistor | RGB green channel |
| BLUE | D11 through 220 ohm resistor | RGB blue channel |
| RGB common cathode | GND | Shared cathode connection |
| All IR sensor VCC | 5V | Sensor power |
| All IR sensor GND | GND | Common ground |

# Wiring instructions

## IR sensors

Each IR sensor has VCC, GND, and OUT pins. Connect VCC to Arduino 5V and GND to Arduino GND. Connect each OUT pin to the assigned digital input: E1 to D2, E2 to D3, X1 to D4, and X2 to D5.

## Blink LED

Connect the LED anode (long leg) to D8 through a 220 ohm resistor. Connect the cathode (short leg) to GND.

## Common-cathode RGB LED

Connect the common cathode (usually the longest leg) to GND. Connect the red, green, and blue channel legs to D9, D10, and D11 respectively, with one 220 ohm resistor in series with each channel.

# System working principle

The Arduino reads the sensors as digital inputs. In this sketch, a LOW reading means that the corresponding IR sensor is detecting an interruption. The entry sensors are arranged in order E1 then E2; the exit sensors are arranged X1 then X2.

| Movement | Required order | Count result |
| --- | --- | --- |
| Entry | E1 detector, then E2 confirmer within 5 seconds | Increase by 1 |
| Exit | X1 detector, then X2 confirmer within 5 seconds | Decrease by 1 when count is above 0 |
| Unconfirmed movement | Confirmer is not detected before the 5-second window expires | Count unchanged; BLINK_LED double-blinks |

When a sequence is confirmed, BLINK_LED flashes once and the RGB status is updated. An exit cannot reduce the count below zero. The Serial Monitor reports the current count at 9600 baud.

# Five-second validation

When E1 or X1 is first detected, the sketch saves the current millis() value as the start time for that direction. It waits for the corresponding confirmer. If E2 or X2 is detected before more than 5,000 milliseconds have elapsed, the movement is accepted. If the window expires first, the stage is cleared, the count does not change, and BLINK_LED flashes twice.

# Crowd status indicators

| Crowd count | RGB output | Meaning |
| --- | --- | --- |
| 0-5 | Green | Low crowd |
| 6-10 | Blue | Medium crowd |
| 11-19 | Solid red | High crowd |
| 20 or more | Blinking red | Overcrowding alert |

The count LED is a separate indicator: it flashes once after a confirmed entry or exit and twice after a failed confirmation.

# Non-blocking red alert

At a count of 20 or more, the sketch calls blinkRedRGB_NonBlocking(). This function compares millis() with the previous red transition time and toggles the red channel every 300 milliseconds when the interval has elapsed. Because it does not wait using delay(), the main loop can continue checking sensors and printing the count while the red alert changes state.

The supplied sketch also uses short delay() calls inside blinkOnce() and blinkTwice() for the separate count/rejection LED. Those brief indications pause loop execution while they run; the red crowd-alert flashing itself is millis()-based and non-blocking.

# Project workflow

Connect the four sensor outputs and LED channels to the pins listed in the connection table.

Upload the Arduino sketch and open the Serial Monitor at 9600 baud.

Test entry by interrupting E1 and then E2 within five seconds; confirm that the count increases.

Test exit by interrupting X1 and then X2 within five seconds; confirm that the count decreases but never below zero.

Leave a detector unconfirmed beyond five seconds; verify that the count stays unchanged and the count LED double-blinks.

Observe the RGB status bands, including blinking red when the count reaches 20.

# Viva explanation

“This project counts entries and exits using four IR sensors arranged as detector and confirmer pairs. The second sensor must be activated within five seconds of the first, otherwise the sequence is rejected and indicated by a double blink. A common-cathode RGB LED shows the crowd range. At 20 people, the red channel toggles on a 300 millisecond interval using millis(), allowing the loop to keep checking the sensors during the alert.”

# Appendix A Final Arduino code

The following is the final code provided in the referenced project conversation.

// IR Sensor Pins

#define E1 2

#define E2 3

#define X1 4

#define X2 5

// LED Pins

#define BLINK_LED 8

#define RED 9

#define GREEN 10

#define BLUE 11

int count = 0;

// Timing control

unsigned long entryStartTime = 0;

unsigned long exitStartTime = 0;

const unsigned long CONFIRM_TIME = 5000; // 5 seconds

bool entryStage = false;

bool exitStage = false;

// RED blink control (non-blocking)

unsigned long redBlinkTime = 0;

bool redState = false;

const unsigned long RED_BLINK_INTERVAL = 300;

void setup() {

pinMode(E1, INPUT);

pinMode(E2, INPUT);

pinMode(X1, INPUT);

pinMode(X2, INPUT);

pinMode(BLINK_LED, OUTPUT);

pinMode(RED, OUTPUT);

pinMode(GREEN, OUTPUT);

pinMode(BLUE, OUTPUT);

Serial.begin(9600);

}

void loop() {

/* -------- ENTRY LOGIC -------- */

if (!entryStage && digitalRead(E1) == LOW) {

entryStage = true;

entryStartTime = millis();

}

if (entryStage) {

if (digitalRead(E2) == LOW) {

count++;

entryStage = false;

blinkOnce();

updateRGB();

}

else if (millis() - entryStartTime > CONFIRM_TIME) {

entryStage = false;

blinkTwice();   // failed confirmation

}

}

/* -------- EXIT LOGIC -------- */

if (!exitStage && digitalRead(X1) == LOW) {

exitStage = true;

exitStartTime = millis();

}

if (exitStage) {

if (digitalRead(X2) == LOW) {

if (count > 0) count--;

exitStage = false;

blinkOnce();

updateRGB();

}

else if (millis() - exitStartTime > CONFIRM_TIME) {

exitStage = false;

blinkTwice();   // failed confirmation

}

}

/* -------- OVERCROWD WARNING -------- */

if (count >= 20) {

blinkRedRGB_NonBlocking();

}

Serial.print("Crowd Count: ");

Serial.println(count);

}

/* -------- BLINK LED FUNCTIONS -------- */

void blinkOnce() {

digitalWrite(BLINK_LED, HIGH);

delay(150);

digitalWrite(BLINK_LED, LOW);

}

void blinkTwice() {

for (int i = 0; i < 2; i++) {

digitalWrite(BLINK_LED, HIGH);

delay(120);

digitalWrite(BLINK_LED, LOW);

delay(120);

}

}

/* -------- RGB STATUS -------- */

void updateRGB() {

if (count < 6) {

setColor(LOW, HIGH, LOW);   // Green

}

else if (count < 11) {

setColor(LOW, LOW, HIGH);   // Blue

}

else if (count < 20) {

setColor(HIGH, LOW, LOW);   // Solid Red

}

}

/* -------- NON-BLOCKING RED BLINK -------- */

void blinkRedRGB_NonBlocking() {

unsigned long currentTime = millis();

if (currentTime - redBlinkTime >= RED_BLINK_INTERVAL) {

redBlinkTime = currentTime;

redState = !redState;

digitalWrite(RED, redState);

digitalWrite(GREEN, LOW);

digitalWrite(BLUE, LOW);

}

}

void setColor(bool r, bool g, bool b) {

digitalWrite(RED, r);

digitalWrite(GREEN, g);

digitalWrite(BLUE, b);

}